Experiment 8

Sine/Cosine Wave Generation using DDS Compiler IP

Objectives

- To design and simulate a Direct Digital Synthesizer (DDS) using the DDS Compiler IP core in Xilinx Vivado.

- To generate a sine wave signal with a frequency of 5 MHz using the DDS Compiler IP.

- To generate a cosine wave signal with a frequency of 10 MHz using the DDS Compiler IP.

- To analyze and verify the generated waveforms based on their frequency, amplitude and phase characteristics.

Software Required

Xilinx Vivado Design Suite / Vivado Simulator

Procedure:

Step 1: Project Creation: Open Xilinx Vivado, create a new RTL project, and select the target FPGA device.

Step 2: 5 MHz Sine Wave Generation: Configure the DDS Compiler IP core to generate a 5 MHz sine wave. Create the RTL wrapper and simulate the design using Vivado Simulator.

Step 3: 10 MHz Cosine Wave Generation: Configure the DDS Compiler IP core to generate a 10 MHz cosine wave. Create the RTL wrapper and simulate the design.

Step 4: Waveform Analysis: Observe the simulated sine and cosine waveforms and measure their frequencies, amplitudes, periodicity, and phase relationship.

Step 5: Result Verification: Compare the measured frequencies with the specified values of 5 MHz and 10 MHz. Calculate the percentage deviation and verify that it is less than 5%.

FORMULA TO BE USED:

fout​=(Phase Increment×fclk​​) / 2^(Phase width)

For Sine wave with 5 MHz

fout=13×100 MHz256≈5.08 MHz

For Cosine wave with 10 MHz

fout=13×100 MHz256≈5.08 MHz

Percentage deviation:

% Deviation=fsimulated-fexpectedfexpected×100

Calculation:

1. Sine Wave Generation at 4 MHz

Given: Clock frequency = 80 MHz, Phase width = 32 bits.

Phase Increment = (4 × 10⁶ × 2³²) / (80 × 10⁶)

Phase Increment = 214748365

2. Cosine Wave Generation at 8 MHz

Given: Clock frequency = 80 MHz, Phase width = 32 bits.

Phase Increment = (8 × 10⁶ × 2³²) / (80 × 10⁶)

Phase Increment = 429496730

3. Percentage Deviation

Formula:

Percentage Deviation = (|Measured Frequency − Desired Frequency| / Desired Frequency) × 100

Example for the 4 MHz sine wave:

Desired frequency = 4 MHz; Measured frequency = 3.96 MHz.

Percentage Deviation = (|3.96 − 4| / 4) × 100 = 1%

Example for the 8 MHz cosine wave:

Desired frequency = 8 MHz; Measured frequency = 8.08 MHz.

Percentage Deviation = (|8.08 − 8| / 8) × 100 = 1%

Output

| S.No | Output Parameter | Target / Expected Frequency | Simulated / Measured Frequency | % Deviation |
| --- | --- | --- | --- | --- |
| 1 | Sine Wave Output (m_axis_data_tdata[15:0]) | 5.00 MHz | 5.078 MHz | < 1.6% |
| 2 | Cosine Wave Output (m_axis_data_tdata[31:16]) | 10.00 MHz | 10.156 MHz | < 1.6% |
| 3 | Output Amplitude Dynamic Range | 16-bit Signed (-32768 to +32767) | 16-bit Signed Waveform | 0% Clipping |
| 4 | Phase Accumulator Stability | Continuous Phase Rollover | Smooth Sinusoidal Output | Pass |

Figure 1: Generated sine/cosine wave

Result

The Direct Digital Synthesizer (DDS) was successfully configured and simulated using the DDS Compiler IP core in Xilinx Vivado. The XSIM results verified the generation of a 5 MHz sine wave and a 10 MHz cosine wave from a 100 MHz clock, with actual frequencies of approximately 5.078 MHz and 10.156 MHz, respectively. Both waveforms exhibited less than 5% frequency deviation, confirming the correct operation of the DDS waveform generation system.