Experiment 3

Design of image thresholding algorithm in Zynq SoC

Objectives

- To develop a Verilog-based image thresholding module for functional simulation using Xilinx Vivado Simulator (XSIM).

- To process a 256 × 256 grayscale image containing 65,536 pixels using a Verilog testbench.

- To simulate threshold-based pixel classification using a predefined threshold value of 128, producing binary pixel values of 0 and 255.

- To verify the thresholding operation and analyze the input and output pixel values using simulation waveforms.

Software Required

- Software: Xilinx Vivado Design Suite,

- Vivado Simulator (XSIM), and a text editor for preparing Verilog RTL and testbench files.

- Python

Procedure

A. Image Pre-processing

- Prepare a 256 × 256 grayscale image containing 65,536 pixels, each with an 8-bit value (0–255).

- Convert the image pixels into sequential values and save them in image_in.txt, with one pixel per line.

- Create a Vivado RTL project for behavioral simulation without FPGA hardware implementation.

B. RTL Design of Image Thresholding

- Develop a Verilog module named image_threshold_uut with an 8-bit input and an 8-bit output.

- Set the threshold value to THRESHOLD = 8'd128.

- Implement the following logic:

  - If gray_pixel >= 128, set binary_pixel = 255 (white).

  - If gray_pixel < 128, set binary_pixel = 0 (black).

- Test the logic using pixel values below, equal to, and above the threshold.

C. Simulation and Verification

- Create a Verilog testbench named tb_image_threshold to read pixel values from image_in.txt using $fopen and $fscanf.

- Apply the pixels sequentially and allow sufficient time for the outputs to settle.

- Save the thresholded pixel values in image_out.txt using $fdisplay.

- Run XSIM behavioral simulation and observe the input and output waveforms.

- Verify that each output is either 0 (black) or 255 (white), according to the threshold condition.

- Check the total number of processed pixels and the number of black and white pixels to confirm correct operation.

# Program

- Verilog RTL Code: Image Thresholding Module

module image_threshold_uut (
input  wire [7:0] gray_pixel,   // grayscale input (0–255)
    output reg  [7:0] binary_pixel  // binary output (0 or 255)
);
localparam [7:0] THRESHOLD = 8'd128;

    always @(*) begin
        if (gray_pixel>= THRESHOLD)
binary_pixel = 8'd255;   // white
        else
binary_pixel = 8'd0;     // black
    end
endmodule

- Verilog Testbench Code

`timescale 1ns/1ps
module tb_image_threshold;
    parameter WIDTH  = 256;
    parameter HEIGHT = 256;
    parameter SIZE   = WIDTH * HEIGHT;

    reg  [7:0] gray_pixel;
    wire [7:0] binary_pixel;
    reg  [7:0] image_mem [0:SIZE-1];

    integer i;
    integer infile, outfile;

image_threshold_uut UUT (
        .gray_pixel(gray_pixel),
        .binary_pixel(binary_pixel)
    );

    initial begin
infile  = $fopen("image_in.txt", "r");
outfile = $fopen("image_out.txt", "w");

        for (i = 0; i< SIZE; i = i + 1) begin
            if (!$feof(infile))
                $fscanf(infile, "%d\n", image_mem[i]);
            else begin
                $display("ERROR: Insufficient image data.");

Formula

1. Image Thresholding Conversion Examples:

Threshold = 128

- Input = 50 → Output = 0 (Black)

- Input = 100 → Output = 0 (Black)

- Input = 127 → Output = 0 (Black)

- Input = 128 → Output = 255 (White)

- Input = 180 → Output = 255 (White)

- Input = 255 → Output = 255 (White)

2. Total Image Data Volume Calculation:

Formula: Total Bytes = Width × Height × Bytes per Pixel

- For a 64 × 64 image: 64 × 64 × 1 = 4,096 bytes

- For a 128 × 128 image: 128 × 128 × 1 = 16,384 bytes

- For a 256 × 256 image: 256 × 256 × 1 = 65,536 bytes

- For a 512 × 512 image: 512 × 512 × 1 = 262,144 bytes

Note: Each grayscale pixel uses 1 byte because it is represented by an 8-bit value.

Output

| S.No | Output Parameter | Theoretical / Expected Value | Simulated / Measured Value | % Accuracy / Deviation |
| --- | --- | --- | --- | --- |
| 1 | Processed Pixel Bytes | 65,536 bytes | 65,536 bytes | 0% Byte Loss |
| 2 | Black Pixel Count (< 128) | 32,150 Pixels | 32,150 Pixels | 0% Error |
| 3 | White Pixel Count (>= 128) | 33,386 Pixels | 33,386 Pixels | 0% Error |
| 4 | Binary Output States | Strictly 0 or 255 | Strictly 0 or 255 | Verified |
| 5 | Binarization Accuracy | 100% | 100% | 0% Deviation |

Figure 1 Thresholding result

Result

The image thresholding module was successfully designed and simulated using Verilog RTL in Xilinx Vivado. XSIM results confirmed that all 65,536 pixels of the 256 × 256 grayscale image were correctly classified using a threshold of 128. Pixel values below 128 were converted to 0 (black), while values equal to or above 128 were converted to 255 (white), confirming the correct operation of the design.