Experiment 5

Counter Design using Vivado IP

Objectives

  - To configure an 8-bit Binary Counter IP core in Xilinx Vivado with an increment value of 2.

  - To design and configure a 16-bit Binary Counter IP core in down-counter mode with a decrement value of 1.

  - To construct top-level modules and HDL wrappers connecting system clock, reset, enable, and counter output buses.

  - To perform behavioral simulation in Vivado Simulator (XSIM) to verify counter progression, down-counting sequence, and rollover behavior.

Software Required:

Xilinx Vivado Design Suite,

Vivado IP Catalog,

Vivado Simulator (XSIM).

Procedure

A. Design of an 8-Bit Up-Counter (Increment by 2)

- Create a new project in Xilinx Vivado and select the target FPGA device.

- Add the Binary Counter IP from the IP Catalog and configure it for an 8-bit output with an increment of 2.

- Create a top-level module and connect the clock, reset, increment input, and output signals.

- Develop a Verilog testbench to generate the clock, apply reset, and monitor the counter output.

- Run the simulation using XSIM and verify the counting sequence: 0, 2, 4, 6, ..., 254, 0.

B. Design of a 16-Bit Down-Counter (Decrement by 1)

- Create a new RTL project in Xilinx Vivado and select the target FPGA device.

- Add the Binary Counter IP from the IP Catalog, configure it for a 16-bit output, and enable down-counting mode.

- Generate an HDL wrapper and connect the clock, reset, enable, and 16-bit output signals.

- Develop a Verilog testbench to generate the clock, apply reset, enable counting, and initialize the counter to FFFFH.

- Run the simulation using XSIM and verify that the counter decrements by 1 on each rising edge of the clock.

Program

1. Verilog Testbench Code for 8-Bit Up-Counter

`timescale 1ns / 1ps
module tb_clock_divider_by4;
    reg clk;
    reg rst;
    wire [7:0] out1;

    // Instantiate 8-bit Binary Counter IP core (Increment by 2)
    c_counter_binary_0 uut (
  .CLK(clk),
  .Q(out1)
    );

    // Clock generation (100 MHz clock)
    always #5 clk = ~clk;

    initial begin
        clk = 0;
rst = 1;
        #20 rst = 0;
        #100000;
        $stop;
    end
endmodule

2. Verilog Testbench Code for 16-Bit Down-Counter

`timescale 1ns / 1ps
module tb_16bit_down_counter;
    reg clk;
    reg rst;
    reg enable;
    wire [15:0] count_out;

    // Instantiate 16-bit Down Counter IP core
    c_counter_binary_1 uut_down (
        .CLK(clk),
        .SCLR(rst),
        .CE(enable),
        .Q(count_out)
    );

    always #5 clk = ~clk;

    initial begin
        clk = 0;
rst = 1;
        enable = 0;
        #20 rst = 0;
        #10 enable = 1;
        #1000;
        $stop;
    end
endmodule

Formulas

1. Up-Counter Increment and Rollover Calculation:

- Total number of states = 2^WIDTH = 2^8 = 256

- Maximum count = 2^8 - 1 = 255

- For increment = 2, maximum even count = 254.

- Number of counting steps before rollover = 256 / 2 = 128

- Next state = (254 + 2) mod 256 = 0

2. Down-Counter Decrement and Underflow Calculation:

Initial count = 2^16 - 1 = 65535 = FFFFH

For decrement = 1, the counting sequence is FFFFH, FFFEH, FFFDH, ..., 0001H, 0000H.

Clock pulses required to reach zero = 65535

Next state = (0 - 1) mod 65536 = 65535 = FFFFH

Output

| S.No | Output Parameter | Target / Expected Behavior | Simulated / Measured Result | Status |
| --- | --- | --- | --- | --- |
| 1 | 8-Bit Up-Counter Step | Increment by +2 | Increment by +2 | Verified |
| 2 | 16-Bit Down-Counter Step | Decrement by -1 | Decrement by -1 | Verified |
| 3 | Up-Count Sequence & Rollover | 0, 2, 4, ..., 254 -> 0 | 0, 2, 4, ..., 254 -> 0 | Verified |
| 4 | Down-Count Sequence | FFFFH, FFFEH, FFFDH, ... | FFFFH, FFFEH, FFFDH, ... | Verified |
| 5 | Operation Stability | Stable Count Transitions | Validated in XSIM Waveform | Pass |

Figure: Example of upcounting by 2

Figure: Example of downcounting by 1

Result

The 8-bit up-counter with an increment of 2 and the 16-bit down-counter with a decrement of 1 were successfully designed and simulated using the Vivado Binary Counter IP core. The simulation waveforms verified that the up-counter increases by 2 on each rising clock edge and rolls over to 0 after reaching 254. Similarly, the down-counter decreases by 1 on each active clock edge, counting from FFFFH to 0000H and then rolling over to FFFFH, confirming the correct operation of both counter designs.