Experiment 1

Design of Clock Divider Circuit using RTL

Objectives:

To design and verify a clock divider by two circuit using RTL (Verilog HDL) that generates a 50 MHz output clock from a 100 MHz input base clock.

To design and verify a clock divider by four circuit using RTL (Verilog HDL) that generates a 25 MHz output clock from a 100 MHz input base clock.

To implement flip-flop toggle logic for divide-by-2 operation and a 2-bit synchronous counter for divide-by-4 operation with active-high asynchronous reset.

To perform behavioral simulation in Vivado Simulator (XSIM), measure output clock periods, and verify frequency division factors.

Software Required:

Xilinx Vivado Design Suite,

Vivado Simulator (XSIM),

PC or laptop.

Procedure:

(a) Generation of 50 MHz Clock from 100 MHz Clock (Divide by 2)

- Create a new RTL project in Xilinx Vivado and select the target FPGA device.

- Create a Verilog module named clock_divider_by2 with clock, reset, and output ports.

- Implement the clock division using toggle logic on the rising edge of the input clock.

- Create a testbench named tb_clock_divider_by2 to generate a 100 MHz clock and apply reset.

- Run XSIM simulation and verify the 50 MHz output waveform.

(b) Generation of 25 MHz Clock from 100 MHz Clock (Divide by 4)

- Create a new RTL project or add a Verilog module named clock_divider_by4.

- Define the clock, reset, and output ports, along with a 2-bit counter register.

- Implement the counter logic to toggle the output clock at the appropriate count to achieve divide-by-4 operation.

- Create a testbench named tb_clock_divider_by4 to generate a 100 MHz clock and apply reset.

- Run XSIM simulation and verify the 25 MHz output waveform.

Program

- Clock Divider by Two Module

module clock_divider_by2 (
input  wire clk,
input  wirerst,
    output reg  clk_out
);
    always @(posedge clk or posedgerst) begin
        if (rst)
clk_out<= 1'b0;
        else
clk_out<= ~clk_out;
    end
endmodule

- Testbench Program for Clock Divider by Two

timescale 1ns / 1ps
module tb_clock_divider_by2;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by2 uut (
  .clk(clk),
  .rst(rst),
  .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
rst = 1;
        #20 rst = 0;
        #100;
        $stop;
    end
endmodule

- Clock Divider by Four Module

module clock_divider_by4 (

input  wire clk,

input  wirerst,

output reg  clk_out

);

reg [1:0] count;

always @(posedge clk or posedgerst) begin

if (rst) begin

count   <= 2'b00;

clk_out<= 1'b0;

end else begin

count <= count + 1'b1;

if (count[0] == 1'b1)

clk_out<= ~clk_out;

end

end

endmodule

- Testbench Program for Clock Divider by Four

`timescale 1ns / 1ps
module tb_clock_divider_by4;
    reg clk;
    reg rst;
    wire clk_out;

    clock_divider_by4 uut (
  .clk(clk),
  .rst(rst),
  .clk_out(clk_out)
    );

    always #5 clk = ~clk; // 100 MHz clock generation

    initial begin
        clk = 0;
rst = 1;
        #20 rst = 0;
        #1000;
        $stop;
    end
endmodule

Output

| S.No | Output Parameter | Theoretical / Expected Value | Simulated / Measured Value | % Deviation |
| --- | --- | --- | --- | --- |
| 1 | Input Clock Period (clk) | 10.0 ns | 10.0 ns | 0% |
| 2 | Divide-by-2 Frequency (clk_out1) | 50.0 MHz | 50.0 MHz | 0% |
| 3 | Divide-by-2 Clock Period | 20.0 ns | 20.0 ns | 0% |
| 4 | Divide-by-4 Frequency (clk_out2) | 25.0 MHz | 25.0 MHz | 0% |
| 5 | Divide-by-4 Clock Period | 40.0 ns | 40.0 ns | 0% |
| 6 | Output Duty Cycle | 50% | 50% | 0% |

Calculation

1. Clock Period Calculation:

Formula: T = 1 / f

Input frequency = 100 MHz

T_in = 1 / (100 × 10⁶) = 10 ns

2. Frequency Division Calculation:

Formula: f_out = f_in / N

- Divide-by-2:

f_out1 = 100 / 2 = 50 MHz

T_out1 = 1 / (50 × 10⁶) = 20 ns

- Divide-by-4:

f_out2 = 100 / 4 = 25 MHz

T_out2 = 1 / (25 × 10⁶) = 40 ns

3. Percentage Frequency Deviation:

Formula: Deviation (%) = |(f_simulated − f_theoretical) / f_theoretical| × 100

- Divide-by-2 example: Simulated frequency = 49.8 MHz

Deviation = |(49.8 − 50) / 50| × 100 = 0.4%

- Divide-by-4 example: Simulated frequency = 24.9 MHz

Deviation = |(24.9 − 25) / 25| × 100 = 0.4%

Figure 1: Clock division by 2

Figure 2: Clock division by 4

Result

The divide-by-2 (50 MHz) and divide-by-4 (25 MHz) clock divider circuits were successfully designed and simulated using Verilog RTL in Xilinx Vivado. XSIM results verified that the 100 MHz input clock was correctly divided to produce output frequencies of 50 MHz and 25 MHz, with corresponding periods of 20 ns and 40 ns, confirming the correct operation of both circuits.